01/16/2025:

- Soldered header pins to PCB and connected PCB to rpi and bluepill - it fits!!

01/17/2025:

- Power levels seem to be correct throughout the PCB

  - Checked 3.3V and 5V spots with multimeter

- I need standoffs for rpi hat

- ANOTHER GOAL: make case for rpi with hat and bluepill??

- Expected output with original setup:

- New setup:

- Something is wrong with R6, it’s not adjusting gain of amplified signal

  - R7 works with adjusting DC offset though

  - Teddy’s idea: use multimeter to see if R6 is adjusting resistance

- Problem: Peaks and dips of amplified signal have weird fluctuations

- Python script successfully ran!

  - Problem listed before is very apparent in graph.

  - But I’m still happy about the success of everything interacting correctly!!

    - Ofc the issue is with the amplifier RIP

- Amplifier pots:

  - 104: 11.47k

  - 103: 10.42k

- PCB pots originally:

  - 104: 78.4k

  - 103: 9.78k

- PCB pots new:

  - 104: 13.25k

- R6 Gain pot does work!! It was just way too high before

- Dr. Scott said he doesn’t necessarily trust the previous group’s ripple voltage graph, so maybe mine could be correct

02/03/2025:

- Cant get the ST-link thing to work, whenever i try to run it, i keep getting this:

  - I remember i had this and fixed it at some point, but idk how. I’ll have to mess with it more later for the documentation

- After messing with the st link, the bluepill still works, i assume bc i never actually uploaded anything new to it

- I adjusted the ECE_449_RW_to_CSV.py script to add pseudo-live updates

  - Implemented for loop to repeat sampling and plot creating a finite, defined number of times

  - GOAL: i’d like to make this better/more visually appealing, so look into it further perhaps

- I really don’t have much to do this semester besides documentation/report writing and this pseudo-live updates which i did today, so maybe i should revisit doubling the Bluepill samplerate

02/06/2025:

- Met with dr scott, review meeting notes and prepare for meeting on tuesday

02/10/2025:

- Prep for dr scott meeting:

  - Previous capstone wiki contains info on Blue Pill ADC calculations and what the numbers mean

02/11/2025:

- More prep for dr scott meeting:

  - Adc.c:

    - Sample rate is set to 50 kHz with math

      - Buck converter has 50 kHz frequency

    - setTimerFreq: arr = 1439 ??

  - DMA : direct memory access - used for ADC and UART

- Investigate if double sample rate with blue pill is possible:

  - “Interleaved sample and hold” Blue Pill documentation ADC pg 20

  - https://community.st.com/t5/stm32-mcus-products/stm32f103c8t6-fast-interleaved-adc-gives-samples-in-wrong-order/td-p/339103

    - STM32F108C6

  - https://stackoverflow.com/questions/54024674/stm32f3-dual-adc-with-interleaved-mode

    - Uses different board

  - https://www.youtube.com/watch?v=AloHXBk6Bfk

    - Uses different board

  - https://community.st.com/t5/stm32-mcus/adc-interleaved-mode-1-2-nbsp-how-to-improve-adc-sampling-rate/ta-p/49295

    - https://community.st.com/t5/stm32-mcus/adc-interleaved-mode-2-2-nbsp-hands-on-stm32cubeide-project/ta-p/49715

    - Uses different board

  - https://www.youtube.com/watch?v=nUejeLYKmfw

    - Uses different board

02/12/2025:

- Most links from yesterday use different boards so different settings are available

- https://github.com/codenuke/STM32-Library-V3.5/blob/master/Project/STM32F10x_StdPeriph_Examples/ADC/RegSimul_DualMode/main.c

  - STM32F10x ✅

  - Possible starting point for code

- STM32F103C8T6:

  - https://community.st.com/t5/stm32-mcus-products/stm32f103c8t6-fast-interleaved-adc-gives-samples-in-wrong-order/td-p/339103

- STM32F10 interleaved ADC, stm32 forum pages:

  - https://community.st.com/t5/stm32-mcus-products/problem-in-using-dma-for-adc-fast-interleaved-mode-in/td-p/534488

  - https://community.st.com/t5/stm32-mcus-products/adc-stm32f1-interleaved/td-p/409387

- Ever since Dr. Scott came in and looked at/messed with my setup, the amplification of the signal has been too low. I’m going to try to adjust the pot again, but what the heck

  - I adjusted the pots and got it to about the size of the ideal path

02/18/2025:

- Think about future work for Dr. Scott potential research student continuing this project

02/26/2025:

- I think i’m missing a resistor at the output of the amplifier omg

  - Seen in the schematic here on the output of the amp

  - But not in my design here

- Try adding the resistor to the output tomorrow? I got a 1k ohm resistor and put in near my PCB on rPi

02/27/2025:

- Before adding 1 kOhm resistor to amp output:

- After adding the 1 kOhm resistor:

- I don’t observe a significant enough effect from the 1 kOhm resistor

03/06/2025:

- To do:

  - Verify length of OSGC presentation

  - Add resistor at end of amp?

  - Start writing OSGC paper

  - Start OSGC presentation